VERA-TDMA

Centralized TDMA Schedule Planner, Optimizer & EMANE Integration

VERA-TDMA is a centralized Python-based TDMA scheduling system that converts radio coordinates into a conflict-free time-slot schedule using graph algorithms, distance-2 interference modeling, heuristic optimization, and exact verification.

The project consists of two parts:

- Part 1 — Network Brain: Mandatory centralized Python schedule optimizer.

- Part 2 — Physical Emulator: EMANE-based schedule enforcement and packet-level validation approach.

1. Problem

In a TDMA network, radios share a common frequency by transmitting during assigned time slots. Interfering radios cannot transmit simultaneously, while sufficiently separated radios can reuse the same slot.

VERA-TDMA models this as a graph-coloring problem using a 500-meter communication range.

The scheduler must:

- Prevent direct-link interference.

- Prevent two-hop/hidden-terminal interference.

- Enable spatial slot reuse.

- Minimize the TDMA frame length.

- Generate a verifiable Slot × Node Boolean matrix.

2. Part 1 — Network Brain

Communication Graph

The network is represented as:

G = (V, E)

where:

V = set of radio nodes

E = set of communication links

For two nodes u and v, their Euclidean distance is:

d(u,v) = sqrt((x_u - x_v)^2 + (y_u - y_v)^2)

A communication link exists when:

d(u,v) <= R

where:

R = 500 meters

3. Distance-2 Conflict Graph

Direct links alone cannot model hidden-terminal interference. Therefore, VERA-TDMA constructs a distance-2 conflict graph:

G2 = (V, E2)

Two nodes are considered conflicting when their shortest-path distance in the communication graph is at most two hops.

1-hop  -> different slots

2-hop  -> different slots

>2-hop -> slot reuse permitted

The slot assignment is formulated as graph coloring.

The coloring function is:

c : V -> {0, 1, ..., K-1}

subject to:

(u,v) in E2  =>  c(u) != c(v)

The objective is:

minimize K

where K is the number of unique TDMA slots.

4. DSATUR Scheduling

The initial schedule is generated using a DSATUR-based graph-coloring heuristic.

The scheduling process is:

Select most constrained node

↓

Identify slots used by conflicting nodes

↓

Assign smallest legal slot

↓

Repeat until all nodes are scheduled

DSATUR prioritizes nodes according to saturation degree, with node degree used as a tie-breaker.

This produces a valid initial coloring while allowing spatial reuse.

5. Schedule Optimization

The initial DSATUR schedule is further evaluated using local optimization.

Two optimization mechanisms are used:

Class Elimination

Attempts to eliminate an entire slot/color class by moving its nodes into other legal slots.

Kempe-Chain Refinement

Attempts valid recoloring transformations between two color classes.

Every candidate modification is accepted only if all distance-2 conflict constraints remain satisfied.

6. Exact Optimization

Minimum graph coloring is computationally difficult, so the system combines heuristic optimization with an exact solver for bounded instances.

The solver establishes:

Lower Bound <= Optimal Chromatic Number <= Upper Bound

For the evaluated topology:

Lower Bound = 9

Upper Bound = 9

Therefore:

Lower Bound = Upper Bound = 9

and:

Optimality Gap = 0

Hence:

Chromatic Number = 9

The 9-slot schedule is therefore proven optimal for the evaluated 16-node topology under the defined distance-2 conflict model.

7. Verified 16-Node Result

| Metric | Result |
| --- | --- |
| Total Nodes | 16 |
| Communication Links | 42 |
| Distance-2 Conflict Edges | 90 |
| Initial DSATUR Frame | 9 |
| Optimized Frame | 9 |
| Exact Lower Bound | 9 |
| Exact Upper Bound | 9 |
| Optimality Gap | 0 |
| Conflict Violations | 0 |
| Verification | PASS |

Node → Slot Mapping

Node_01 -> Slot 8

Node_02 -> Slot 4

Node_03 -> Slot 5

Node_04 -> Slot 8

Node_05 -> Slot 6

Node_06 -> Slot 0

Node_07 -> Slot 1

Node_08 -> Slot 6

Node_09 -> Slot 7

Node_10 -> Slot 2

Node_11 -> Slot 3

Node_12 -> Slot 7

Node_13 -> Slot 8

Node_14 -> Slot 4

Node_15 -> Slot 5

Node_16 -> Slot 8

Spatial reuse is demonstrated by multiple non-conflicting nodes sharing the same slot.

8. Independent Verification

The final schedule is independently verified for:

- Coordinate integrity.

- Node count.

- Direct conflicts.

- Two-hop conflicts.

- Slot assignment validity.

- Total violations.

For the evaluated topology:

Direct conflicts checked  : 42

Two-hop conflicts checked : 90

Violations                : 0

Verification              : PASS

Optimization audit:

Class elimination attempts : 9

Accepted eliminations      : 0

Kempe attempts             : 42

Accepted Kempe moves       : 0

The initial DSATUR solution already used the exact minimum of 9 slots.

9. Schedule Matrix

The scheduler produces an explicit Slot × Node Boolean Matrix.

1 = Node assigned to slot

0 = Node not assigned to slot

Slot 0 -> 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0

Slot 1 -> 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0

Slot 2 -> 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0

Slot 3 -> 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0

Slot 4 -> 0 1 0 0 0 0 0 0 0 0 0 0 0 1 0 0

Slot 5 -> 0 0 1 0 0 0 0 0 0 0 0 0 0 0 1 0

Slot 6 -> 0 0 0 0 1 0 0 1 0 0 0 0 0 0 0 0

Slot 7 -> 0 0 0 0 0 0 0 0 1 0 0 1 0 0 0 0

Slot 8 -> 1 0 0 1 0 0 0 0 0 0 0 0 1 0 0 1

10. Part 2 — EMANE Physical Emulator

Part 2 extends the verified Part 1 schedule into an EMANE TDMA Radio Model environment.

The integration architecture is:

Python Schedule Optimizer

↓

Verified Schedule Matrix

↓

Python → EMANE Integration Bridge

↓

EMANE TDMA Schedule

↓

Virtual Radio Nodes

↓

Packet-Level Validation

The EMANE configuration is designed around:

- 1 ms TDMA slots.

- Configurable frame length.

- Frequency/channel configuration.

- Virtual radio nodes.

- Schedule enforcement.

For the demonstrated 9-slot schedule:

Slot Duration = 1 ms

Frame Length = 9 slots

Frame Duration = 9 ms

The integration bridge consumes the schedule generated by Part 1 and converts the node-to-slot mapping into the corresponding EMANE schedule/event representation.

Part 2 does not independently calculate another schedule.

11. EMANE Validation Approach

The intended validation experiments are:

Test 1 — Scheduled Transmission

Node transmits during assigned slot

↓

Transmission permitted

↓

Packet delivered

Test 2 — Unscheduled Transmission

Node attempts transmission outside assigned slot

↓

Transmission suppressed

Test 3 — Conflicting Nodes

Distance-2 conflicting nodes

↓

Different assigned slots

↓

No simultaneous scheduled transmission

Test 4 — Spatial Reuse

Non-conflicting nodes

↓

Same slot

↓

Simultaneous transmission permitted

Part 2 is treated as an integration/prototype component because the problem statement gives greater weight to the mandatory Part 1 implementation and states that a complete Part 2 result is not required.

12. Project Structure

VERA-TDMA/

├── data/

├── experiments/

├── src/

├── tests/

├── docs/

│   └── VERA-TDMA.docx

├── presentation/

│   └── PPT_VERA-TDMA.pdf

├── results_16_nodes.json

├── config.yaml

├── PATCH_NOTES.md

├── pyproject.toml

├── README.md

└── .gitignore

13. Installation and Testing

python -m venv .venv

.\.venv\Scripts\Activate.ps1

pip install -e .

python -m pip install pytest

python -m pytest -q

The generated JSON result contains:

- Exact solver result.

- Lower/upper bounds.

- Optimality gap.

- Communication links.

- Distance-2 conflicts.

- Node-to-slot mapping.

- Boolean schedule matrix.

- Verification status.

- Optimization audit.

- Input and schedule hashes.

14. Research Basis

The project is based on established research in TDMA scheduling, graph coloring, distance-2 coloring, wireless interference modeling, and scheduling heuristics.

Key references:

- Brélaz, D., “New Methods to Color the Vertices of a Graph,” Communications of the ACM, 1979.

- Ergen, S. and Varaiya, P., “TDMA Scheduling Algorithms for Wireless Sensor Networks,” Wireless Networks, 2010.

- Kuhn, F., “Local Multicoloring Algorithms: Computing a Nearly-Optimal TDMA Schedule in Constant Time,” STACS, 2009.

- EMANE Project, TDMA Radio Model Documentation.

15. Limitations

The core optimizer uses a deterministic distance-2 graph interference model.

Therefore, the primary guarantee is:

The generated schedule is conflict-free under the

defined distance-2 interference model.

It does not directly model:

- SINR.

- Multipath propagation.

- Fading.

- Antenna characteristics.

- Capture effects.

- Aggregate physical interference.

EMANE integration provides the next layer for evaluating schedule enforcement in a network-emulation environment.

sss

16. Project Status

Part 1 — Network Brain

- Coordinate-based topology

- 500 m communication graph

- Distance-2 conflict graph

- DSATUR scheduling

- Spatial reuse

- Class elimination

- Kempe refinement

- Exact bounded solver

- Independent verification

- Boolean schedule matrix

- JSON results

- 16-node evaluation

Part 2 — EMANE

- Integration architecture

- TDMA configuration approach

- 1 ms slot design

- Python-to-EMANE bridge design

- Packet validation methodology

- Complete EMANE deployment

- End-to-end packet experiment

VERA-TDMA provides a complete and independently verified TDMA scheduling engine, together with a defined EMANE integration path for physical schedule enforcement.